L20A
DOORS/SHUTTERS/HATCHES


To be read with Preliminaries / General Conditions

545 RELOCATABLE PARTITION SYSTEM TO ..........................


Supplier:

Multispace Systems Ltd., Unit 12, Driffield Business Centre, Scotchburn Garth, Skerne Road, East Yorkshire, YO25 6EF, Tel: (01377) 250295, Fax: (01377) 250618

Product Reference:

Acoustic Type 100 Movable Wall

Strength to BS5234-2 Grade Heavy Duty

Sound Insulation: Rw 37, 40, 44, 47, or 50 dB in accordance with BS2750: Part 3: 1980 and ISO 140/III – 1978

Partition abutments:


Head:


1.
Directly fixed to a suitable supporting structure via M8 Coach Screws

2.
Steel Beam (Track to be fitted using M10 drop rod suspension system)

3.
Concrete Soffit (Track to be fitted using M10 drop rod suspension 
system)


Deflection: +/- 5 mm

Floor Finish: TBA

Wall/Vertical Structure: TBA

Framing: Exposed aluminium perimeter frame which provides exceptional durability.

Seals: Top and bottom manually operated box seals with positive male / female vertical profiles. (Rubber inserted nosings are not acceptable)

Solid Panels: 12, 15, or 18 mm Chipboard with Rockwool and acoustic core materials.

Finish: Choice from Veneer, Vinyl, Laminate, Spray Paint or Melamine. 

Additional Notes

Void between head track and soffit should be filled with an acoustic baffle, constructed to an equivalent acoustic rating as the movable wall.

Head track can be incorporated into the ceiling / bulkhead construction utilising the unique head track section which has a built in support lip.

One or Two Point head track to be powder coated White.

System to comprise ...... No Basic Panels plus 1 No Telescopic Panel all .............mm wide.

Partition is to be lockable when parked, to avoid unauthorised use.

For ease of future maintenance all panels must be capable of being dismantled, repaired and re-assembled on site without the need for returning to the factory.

Additionally, the panels should also be capable of being removed from the head track without removing a section of head track.

