L20A
DOORS/SHUTTERS/HATCHES


To be read with Preliminaries / General Conditions

545 RELOCATABLE PARTITION SYSTEM TO ..........................


Supplier:

Multispace Systems Ltd., Unit 12, Driffield Business Centre, Scotchburn Garth, Skerne Road, East Yorkshire, YO25 6EF, Tel: (01377) 250295, Fax: (01377) 250618

Product Reference:

Glazed GS100 Movable Wall (Double Glazed)

Strength to BS5234-2 Grade Heavy Duty

Sound Insulation: Up to 35 dB, on site performance.

Partition abutments:


Head:


1.
Directly fixed to a suitable supporting structure via M8 Coach Screws

2.
Steel Beam (Track to be fitted using M10 drop rod suspension system)

3.
Concrete Soffit (Track to be fitted using M10 drop rod suspension 
system)


Deflection: +/- 5 mm

Floor Finish: TBA

Wall/Vertical Structure: TBA

Framing: Semi-Concealed aluminium perimeter frame which protects the edge of the panel without lipping on to the face. Framing is powder coated to match the glazing.

Seals: Top and bottom manually operated box seals with positive male / female vertical profiles. (Rubber inserted nosings are not acceptable)

Panels: Fully glazed with a painted glass border that conceals all the mechanics of the panel. Integral blinds can be incorporated, if required.

Additional Notes

Two Point head track to be powder coated.

System to comprise ...... No Basic Panels plus 1 No Communication Panel.

Partition is to be lockable when parked, to avoid unauthorised use.

